CPE301 – SPRING 2020

# Design Assignment 3A

Student Name: Jason Villanueva

Student #: 2001255420

Student Email: villaj21@unlv.nevada.edu

Primary Github address: https://github.com/jasonvillanuevagit/submission_designAssignments-

Directory: https://github.com/jasonvillanuevagit/submission_designAssignments-/tree/master/DesignAssignment3A

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7.0	Atmega328PB-Xmini PC

- Assembler

- Simulator

- Debugger

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

DA_3A T1 (C)

#define BAUD 9600

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <util/setbaud.h>

#include <avr/interrupt.h>

#include <stdlib.h>

#include <stdio.h>

//FUNCTION TO INITIALIZE USART

void USART_init(void){

UBRR0H = UBRRH_VALUE;

UBRR0L = UBRRL_VALUE;

UCSR0C = _BV(UCSZ01) | _BV(UCSZ00); //8-BIT DATA

UCSR0B = _BV(RXEN0) | _BV(TXEN0);	//ENABLE RX AND TX

}

//FUNCTION THAT READS IN 1 CHAR AT A TIME

void USART_tx_string(char*data){

//CONTROL ENTERS WHILE DATA REG NOT EMPTY

while((*data!='\0')){

//WAIT FOR BUFFER REGISTER TO CLEAR

while(!(UCSR0A & (1 << UDRE0)));

UDR0 = *data;//REGESTER EQUALS DATA

data++;//DATA MOVES POSITION

}

}

int main(void){

USART_init();//Initialize USART

int num = 2303;//Initialize integer value

char char_array [16];//Character array of 16 for integer

char char_array2[16];//Character array of 16 for float

while (1){

//Initialization of float value

volatile float float_value = num * 3.2;

//To generate random number

num = (((num * 3) % 100) + 2);

itoa (num, char_array, 10);

snprintf(char_array2,sizeof(char_array2), "%f\r\n", float_value);

//Prints string Jason Villanueva

USART_tx_string("Jason Villanueva");

USART_tx_string(" ");

//Prints integer value

USART_tx_string(char_array);

USART_tx_string(" ");

//Prints floating value

USART_tx_string(char_array2);

USART_tx_string("\n");

USART_tx_string("\n");

_delay_ms(1000);

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

DA_3A T2 (C)

#define BAUD 9600

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <util/setbaud.h>

#include <avr/interrupt.h>

#include <stdlib.h>

#include <stdio.h>

//"Global variable"

volatile uint8_t Overflow;

//FUNCTION TO INITIALIZE USART

void USART_init(void){

UBRR0H = UBRRH_VALUE;

UBRR0L = UBRRL_VALUE;

UCSR0C = _BV(UCSZ01) | _BV(UCSZ00); //8-BIT DATA

UCSR0B = _BV(RXEN0) | _BV(TXEN0);	//ENABLE RX AND TX

}

//FUNCTION THAT SETS TIMER/INTERRUPT REGISTERS

void set(){

TCCR0A = 0;//NORMAL MODE OPERATION

TCCR0B  = 0X05;//THE PRESCALER SET TO 1024

TCNT0 = 0X00;//COUNTER VALUE = 0

TIMSK0 = (1<<TOIE0);//ENABLE INTERRUPT

sei();//ENABLE GLOBAL INTERRUPT

}

//INTERUPT SUBROUTINE FOR TIMER 0

ISR(TIMER0_OVF_vect){

Overflow++;//INCREMENT OVERFLOW

}

//FUNCTION THAT READS IN 1 CHAR AT A TIME

void USART_tx_string(char*data){

//CONTROL ENTERS WHILE DATA REG NOT EMPTY

while((*data!='\0')){

//WAIT FOR BUFFER REGISTER TO CLEAR

while(!(UCSR0A & (1 << UDRE0)));

UDR0 = *data;//REGESTER EQUALS DATA

data++;//DATA MOVES POSITION

}

}

int main(void){

USART_init();//Initialize USART

set();//CALL SET FUNCTION

int num = 2303;//Initialize integer value

char char_array [16];//Character array of 16 for integer

char char_array2[16];//Character array of 16 for float

while (1){

//Initialization of float value

volatile float float_value = num * 3.2;

//CONTROL WILL ENTER IF OVERFLOW >= 61 CYCLES ~1sec

if (Overflow >= 61){

//To generate random number

num = (((num * 3) % 100) + 2);

itoa (num, char_array, 10);

snprintf(char_array2,sizeof(char_array2), "%f\r\n", float_value);

//Prints string Jason Villanueva

USART_tx_string("Jason Villanueva");

USART_tx_string(" ");

//Prints integer value

USART_tx_string(char_array);

USART_tx_string(" ");

//Prints floating value

USART_tx_string(char_array2);

USART_tx_string("\n");

USART_tx_string("\n");

//Overflow reset to 0

Overflow = 0;

}

}

}

- SCHEMATICS

N/A

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Task 1 - https://youtu.be/PNXIC-XKTB8

Task 2 - https://youtu.be/WqPRB4P66MI

- GITHUB LINK OF THIS DA

https://github.com/jasonvillanuevagit/submission_designAssignments-/tree/master/DesignAssignment3A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Jason Villanueva