CPE301 – SPRING 2020

# Design Assignment 6

Student Name: Jason Villanueva

Student #: 2001255420

Student Email: villaj21@unlv.nevada.edu

Primary Github address: https://github.com/jasonvillanuevagit/submission_designAssignments-

Directory: https://github.com/jasonvillanuevagit/submission_designAssignments-/tree/master/DesignAssignment6

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7.0	Atmega328PB-Xmini PC	MPU6050 Sensor

- Assembler 		-Multifunction Shield

- Simulator

- Debugger

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define  F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <stdio.h>

#include <math.h>

//TW_CONFIG

#define PRESCALER				1U

#define SCL_FREQ_200kH			200000UL

#define SCL_FREQ_400kH			400000UL

#define TW_TWBR					(((F_CPU/SCL_FREQ_400kH) - 16)/(2*PRESCALER))

#define TW_config_SCL_200kH

#define TW_TWCR_TWINT	(1U<<7)

#define TW_TWCR_TWEA	(1U<<6)

#define TW_TWCR_TWSTA   (1U<<5)

#define TW_TWCR_TWSTO	(1U<<4)

#define TW_TWCR_TWWC	(1U<<3)

#define TW_TWCR_TWEN	(1U<<2)

#define TW_TWCR_TWIE	(1U)

#define TW_TWSR_scaler64 (3U<<0)

#define TW_TWSR_scaler4  (1U<<0)

#define TW_TWSR_scaler1  (0U<<0);

//PWM

#define PWM_8Scalar				0x02

#define PWM_noScalar			0x01

#define PWM_WGM_11_10			0x02

#define PWM_WGM_13_12			0x02

#define PWM_nonInvert			0x02

#define PWM_timerOVR			0x01

//Bool

#define bool uint8_t

#define true 0x01U

#define false 0x00U

#define MPU6050_conversion_accel_2g		16384.0

#define MPU6050_conversion_gyro_250dps	131.0

#define MPU6050_conversion_gyro_2000dps 16.4

#define MPU6050_conversion_radians_to_degrees (180.0/acos(-1))

typedef enum{

MasterTX_Data,

MasterTX_Done,

}status_t;

typedef struct{

volatile int8_t rx[12];

volatile uint8_t rxCounter;

volatile uint8_t tx;

volatile uint8_t slaveAddr;

volatile uint8_t regAddr;

volatile bool busy;

volatile status_t status;

}TW_data_t;

typedef struct{

volatile float X;

volatile float Y;

volatile float Z;

}accel_t;

typedef struct{

volatile float X;

volatile float Y;

volatile float Z;

volatile float xErr;

volatile float yErr;

volatile float zErr;

}gyro_t;

typedef struct{

volatile float curr_Angle;

volatile float prev_Angle;

}tilt_angle_t;

TW_data_t TW_data;

accel_t accel_data;

gyro_t gyro_data;

tilt_angle_t roll_angle;

tilt_angle_t pitch_angle;

volatile bool sampleReady = false;

#define MPU6050_FIFO_RegEn	  TW_send(0x68, 35U, ((1U<<6)|(1U<<5)|(1U<<4)|(1U<<3)))

#define MPU6050_FIFO_EN		  TW_send(0x68, 106U, (1U<<6))

#define MPU6050_CLEAR_STATUS  TW_receive(0x68, 58U, 1)

#define MPU6050_FIFO_CLEAR    TW_send(0x68, 106U, (1U<<2));

//Uart

#define rxEN 1

#define txIEN 1

#define drIEN 1

#define txEN 1

#define eightBit 0x03

#define async 0x00

#define sync 0x01

#define noParity 0x00

#define oneStop 0x00

#define twoStop 0x01

#define chSize8 0x03

#define BAUD 300000UL

#define BAUDPRESCALAR ((F_CPU/(BAUD))/16 - 1) //Register Values

static volatile unsigned int usartBUSY;  //ISR variables

static volatile char* dataString;

static char charConversion[20];

void TW_send(uint8_t slaveAddr, uint8_t regAddr, uint8_t dataTx);

void TW_receive(uint8_t slaveAddr, uint8_t regAddr, uint8_t rxCounter);

void initiate_TW();

void TW_Status_Code_TX();

void TW_Status_Code_RX();

float combineRegisters(int16_t high, int16_t low);

float calculateSensorData(int16_t data, int16_t conversion);

void convert_float_to_string(float floatNum);

void sendString_UART();

void initUART(uint8_t umsel, uint8_t upm, uint8_t usbs, uint8_t ucsz);

void UART_Send_Values();

void calculatePosition();

void inititate_MPU6050();

void initiate_INT0();

void storeSensorValues_Accel();

void storeSensorValues_Gyro();

float calculateAngle_Pitch_Accel();

float calculatetAngle_Roll_Accel();

float calculateAngle_Pitch_Gyro();

float calculatetAngle_Roll_Gyro();

float complementaryFilter(float gyro_angle, float accel_angle, tilt_angle_t* angle);

void calculateGyroError();

void storeGyroError();

void initTimer1PWM(unsigned int top);

void startPWM1();

void sendMessage(char* mess);

void send_accel_values(accel_t*);

void send_gyro_values(gyro_t*);

#define dt 0.0100

#define scale_gyro   .96

#define scale_accel  .02

volatile float temp;

int main(void){

TW_data.rx[0] = 0xFF;

DDRB = (1U << 1); //DDRB.1 out OC1A

initiate_TW();

initUART(async, noParity, oneStop, eightBit); //initiate UART 8-N-1

initiate_INT0();

initTimer1PWM(20000);

sei();

startPWM1(); //start PWM

inititate_MPU6050();

calculateGyroError();

while (1){

if(sampleReady){

sampleReady = false;

storeSensorValues_Accel();

storeSensorValues_Gyro();

send_accel_values(&accel_data);

send_gyro_values(&gyro_data);

pitch_angle.curr_Angle = complementaryFilter(calculateAngle_Pitch_Gyro(), calculateAngle_Pitch_Accel(), &pitch_angle);

convert_float_to_string(pitch_angle.curr_Angle);

sendString_UART();

sendMessage(" ");

roll_angle.curr_Angle  = complementaryFilter(calculatetAngle_Roll_Gyro(), calculatetAngle_Roll_Accel(), &roll_angle);

convert_float_to_string(roll_angle.curr_Angle);

sendString_UART();

sendMessage(" ");

sendMessage("\r\n");

TW_receive(0x68, 58U, 1);

}

}

}

void TW_Status_Code_TX(){

switch(TWSR0 & 0xF8){

case 0x08:

(TWDR0 = (TW_data.slaveAddr<<1));

(TWCR0 = TW_TWCR_TWEN|TW_TWCR_TWINT);

break;

case 0x18:

(TWDR0 = TW_data.regAddr);

TW_data.status = MasterTX_Data;

(TWCR0 = TW_TWCR_TWEN|TW_TWCR_TWINT);

break;

case 0x28:

if(TW_data.status == MasterTX_Data){

(TWDR0 = TW_data.tx);

TW_data.status = MasterTX_Done;

(TWCR0 = TW_TWCR_TWEN|TW_TWCR_TWINT);

}

else{

(TWCR0 = TW_TWCR_TWSTO|TW_TWCR_TWEN | TW_TWCR_TWINT);

TW_data.busy = false;

}

break;

default:

TW_data.busy = false;

}

}

void TW_Status_Code_RX(){

switch((TWSR0 & 0xF8)){

case 0x08:

(TWDR0 = (TW_data.slaveAddr<<1));

(TWCR0 = TW_TWCR_TWEN|TW_TWCR_TWINT);

break;

case 0x18:

(TWDR0 = TW_data.regAddr);

(TWCR0 = TW_TWCR_TWEN|TW_TWCR_TWINT);

break;

case 0x28:

(TWCR0 = TW_TWCR_TWSTA|TW_TWCR_TWEN | TW_TWCR_TWINT);

break;

case 0x10:

(TWDR0 = ((TW_data.slaveAddr<<1) | 0x01));

(TWCR0 = TW_TWCR_TWEN|TW_TWCR_TWINT);

break;

case 0x40:

if(TW_data.rxCounter == 0){

(TWCR0 = TW_TWCR_TWEN|TW_TWCR_TWINT);

}

else{

(TWCR0 = TW_TWCR_TWEA|TW_TWCR_TWEN | TW_TWCR_TWINT);

}

break;

case 0x50:

if(TW_data.rxCounter == 1){

TW_data.rx[TW_data.rxCounter] = (TWDR0);

(TWCR0 = TW_TWCR_TWEN|TW_TWCR_TWINT);

}

else{

TW_data.rx[TW_data.rxCounter] = (TWDR0);

TW_data.rxCounter--;

(TWCR0 = TW_TWCR_TWEA|TW_TWCR_TWEN | TW_TWCR_TWINT);

}

break;

case 0x58:

TW_data.rx[0] =  (TWDR0);

(TWCR0 = TW_TWCR_TWSTO|TW_TWCR_TWEN | TW_TWCR_TWINT);

TW_data.busy = false;

break;

default:

TW_data.busy = false;

}

}

void sendMessage(char* mess){

dataString = mess;

sendString_UART();

}

void send_gyro_values(gyro_t* gyro){

convert_float_to_string(gyro->X);

sendString_UART();

sendMessage(" ");

convert_float_to_string(gyro->Y);

sendString_UART();

sendMessage(" ");

convert_float_to_string(gyro->Z);

sendString_UART();

sendMessage(" ");

}

void send_accel_values(accel_t* accel){

convert_float_to_string(accel->X);

sendString_UART();

sendMessage(" ");

convert_float_to_string(accel->Y);

sendString_UART();

sendMessage(" ");

convert_float_to_string(accel->Z);

sendString_UART();

sendMessage(" ");

}

void calculatePosition(){

if(pitch_angle.curr_Angle < 0){

OCR1A = (uint16_t)(1500 + pitch_angle.curr_Angle*7.5);

}

else{

OCR1A = (uint16_t)(1500 + pitch_angle.curr_Angle*7.5);

}

}

void initiate_INT0(){

DDRD &= ~(1<<2);

PORTD &= ~(1<<2);

EICRA = 0x03;

EIMSK = 0x01;

}

void inititate_MPU6050(){

sampleReady = false;

pitch_angle.prev_Angle = 0;

roll_angle.prev_Angle = 0;

gyro_data.X = 0;

gyro_data.Y = 0;

gyro_data.Z = 0;

TW_send(0x68, 27U, (3U<<3));

TW_send(0x68, 25U, 47U);

TW_send(0x68, 55U, 0x20);

TW_send(0x68, 56U, 0x01);

TW_send(0x68, 0x6B, 0x00);

TW_receive(0x68, 58U, 1);

}

void storeSensorValues_Gyro(){

volatile uint8_t sensorValH;

volatile uint8_t sensorValL;

TW_receive(0x68, 0x43, 1);

sensorValH = TW_data.rx[0];

TW_receive(0x68, 0x44, 1);

sensorValL = TW_data.rx[0];

gyro_data.X = calculateSensorData(combineRegisters(sensorValH, sensorValL)-gyro_data.xErr, MPU6050_conversion_gyro_2000dps);

TW_receive(0x68, 0x45, 1);

sensorValH = TW_data.rx[0];

TW_receive(0x68, 0x46, 1);

sensorValL = TW_data.rx[0];

gyro_data.Y = calculateSensorData(combineRegisters(sensorValH, sensorValL)-gyro_data.yErr, MPU6050_conversion_gyro_2000dps);

TW_receive(0x68, 0x47, 1);

sensorValH = TW_data.rx[0];

TW_receive(0x68, 0x48, 1);

sensorValL = TW_data.rx[0];

gyro_data.Z = calculateSensorData(combineRegisters(sensorValH, sensorValL)-gyro_data.zErr, MPU6050_conversion_gyro_2000dps);

}

void calculateGyroError(){

for(unsigned int i = 0; i < 200; i++){

while(!sampleReady);

sampleReady = false;

storeGyroError();

TW_receive(0x68, 58U, 1);

}

gyro_data.xErr = gyro_data.X/200;

gyro_data.yErr = gyro_data.Y/200;

gyro_data.zErr = gyro_data.Z/200;

gyro_data.X = 0;

gyro_data.Y = 0;

gyro_data.Z = 0;

TW_receive(0x68, 58U, 1);

}

void storeGyroError(){

volatile uint8_t sensorValH;

volatile uint8_t sensorValL;

TW_receive(0x68, 0x43, 1);

sensorValH = TW_data.rx[0];

TW_receive(0x68, 0x44, 1);

sensorValL = TW_data.rx[0];

gyro_data.X += combineRegisters(sensorValH, sensorValL);

TW_receive(0x68, 0x45, 1);

sensorValH = TW_data.rx[0];

TW_receive(0x68, 0x46, 1);

sensorValL = TW_data.rx[0];

gyro_data.Y += combineRegisters(sensorValH, sensorValL);

TW_receive(0x68, 0x47, 1);

sensorValH = TW_data.rx[0];

TW_receive(0x68, 0x48, 1);

sensorValL = TW_data.rx[0];

gyro_data.Z += combineRegisters(sensorValH, sensorValL);

}

void storeSensorValues_Accel(){

volatile uint8_t sensorValH;

volatile uint8_t sensorValL;

TW_receive(0x68, 0x3B, 1);

sensorValH = TW_data.rx[0];

TW_receive(0x68, 0x3C, 1);

sensorValL = TW_data.rx[0];

accel_data.X = calculateSensorData(combineRegisters(sensorValH, sensorValL), MPU6050_conversion_accel_2g);

TW_receive(0x68, 0x3D, 1);

sensorValH = TW_data.rx[0];

TW_receive(0x68, 0x3E, 1);

sensorValL = TW_data.rx[0];

accel_data.Y = calculateSensorData(combineRegisters(sensorValH, sensorValL), MPU6050_conversion_accel_2g);

TW_receive(0x68, 0x3F, 1);

sensorValH = TW_data.rx[0];

TW_receive(0x68, 0x40, 1);

sensorValL = TW_data.rx[0];

accel_data.Z = calculateSensorData(combineRegisters(sensorValH, sensorValL), MPU6050_conversion_accel_2g);

}

float combineRegisters(int16_t high, int16_t low){

return ((high<<8) | low);

}

float calculateSensorData(int16_t data, int16_t conversion){

return ((float)data/(float)conversion);

}

float calculateAngle_Pitch_Accel(){

return (float)(atan(accel_data.Y/sqrt(accel_data.X*accel_data.X + accel_data.Z*accel_data.Z))*MPU6050_conversion_radians_to_degrees);

}

float calculatetAngle_Roll_Accel(){

return (float)(atan(accel_data.X/(sqrtf(accel_data.Y*accel_data.Y + accel_data.Z*accel_data.Z)))*MPU6050_conversion_radians_to_degrees);

}

float calculateAngle_Pitch_Gyro(){

return (gyro_data.X * dt);

}

float calculatetAngle_Roll_Gyro(){

return (gyro_data.Y * dt);

}

float complementaryFilter(float gyro_angle, float accel_angle, tilt_angle_t* angle){

angle->prev_Angle = (float)(((gyro_angle + angle->prev_Angle) * scale_gyro) + (accel_angle * scale_accel));

return angle->prev_Angle;

}

void TW_send(uint8_t slaveAddr, uint8_t regAddr, uint8_t dataTx){

TW_data.busy = true;

TW_data.slaveAddr = slaveAddr;

TW_data.tx = dataTx;

TW_data.regAddr = regAddr;

(TWCR0 = TW_TWCR_TWSTA|TW_TWCR_TWEN);

while(TW_data.busy){

while(!(TWCR0 & (1U<<TWINT))){};

TW_Status_Code_TX();

}

(TWCR0 = 0x00);

}

void TW_receive(uint8_t slaveAddr, uint8_t regAddr, uint8_t rxCounter){

TW_data.busy = true;

TW_data.slaveAddr = slaveAddr;

TW_data.regAddr = regAddr;

TW_data.rxCounter = rxCounter - 1;

(TWCR0 = TW_TWCR_TWSTA|TW_TWCR_TWEN);

while(TW_data.busy){

while(!(TWCR0 & (1<<TWINT))){};

TW_Status_Code_RX();

}

(TWCR0 = 0x00);

}

void initiate_TW(){

TWBR0 = TW_TWBR;

TWSR0 |= TW_TWSR_scaler1;

}

void convert_float_to_string(float floatNum){

snprintf(charConversion, sizeof(charConversion), "%f ", floatNum);

dataString = charConversion;

}

void sendString_UART(){

//USART busy, enable TX and interrupts

usartBUSY = 0x01;

UCSR0B |= ((txIEN << TXCIE0) |

(drIEN << UDRIE0) |

(txEN  << TXEN0));

while(usartBUSY);

}

void initUART(uint8_t umsel, uint8_t upm, uint8_t usbs, uint8_t ucsz){

//initiates UART Baud and sets it to a 8-N-1 setting

UBRR0H = (uint8_t)(0x00);

UBRR0L = (uint8_t)(BAUDPRESCALAR);

UCSR0C = (umsel << UMSEL00) |

(upm   << UPM00)	|

(usbs  << USBS0)	|

(ucsz  << UCSZ00);

}

void initTimer1PWM(unsigned int top){

//inititate PWM

TCCR1A |= (PWM_nonInvert << COM1A0);

ICR1    = (top);

}

void startPWM1(){

//start PWM

TCNT1L = 0x00;

TCNT1H = 0x00;

TCCR1A |= (PWM_WGM_11_10 << WGM10);

TCCR1B |= (PWM_WGM_13_12 << WGM12);

TIMSK1 |= (PWM_timerOVR << TOIE1);

TCCR1B |= (PWM_8Scalar  << CS10);

}

ISR(USART0_UDRE_vect){

//if datastring is not empty send character

//else send disable interrupt and signal when usart is done

if(*dataString != '\0'){

UDR0 = *dataString;

dataString++;

}

else{

UCSR0B &= ~(drIEN << UDRIE0);

usartBUSY = 0x00;

}

}

ISR(TIMER1_OVF_vect){

calculatePosition();

}

ISR(USART0_TX_vect){

//disables TX and

UCSR0B &= ~((txIEN << TXCIE0) |

(txEN  << TXEN0));

}

ISR(INT0_vect){

sampleReady = true;

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

- GITHUB LINK OF THIS DA

https://github.com/jasonvillanuevagit/submission_designAssignments-/tree/master/DesignAssignment6

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Jason Villanueva